# Full Specification Table: Meta Ray-Ban Smart Glasses, Lenovo ThinkReality A3, Microsoft HoloLens 2

| Category | Meta Ray-Ban Smart Glasses | Lenovo ThinkReality A3 | Microsoft HoloLens 2 |
| --- | --- | --- | --- |
| Device Type | Smart glasses (camera + audio) | AR smart glasses (tethered) | Standalone mixed-reality headset |
| Price | ~$299–$379 | ~$1,499 | ~$3,500 |
| Release Year | 2023 (2nd Gen) | 2021 | 2019 |
| Weight | ~48–50 g | ~130 g | ~566 g |
| Dimensions | Eyewear form factor | Glasses form factor | Head-mounted visor |
| Operating System | Meta firmware | Android-based custom OS | Windows Holographic |
| CPU / Processor | Qualcomm Snapdragon (light SoC) | Uses PC/phone CPU | Qualcomm Snapdragon 850 + HPU 2.0 |
| GPU | Integrated mobile GPU | Uses PC/phone GPU | Adreno + custom HPU |
| RAM | Not specified | Uses PC/phone RAM | 4 GB LPDDR4x |
| Storage | Not applicable | Uses PC/phone storage | 64 GB UFS |
| Display Type | None (not AR) | 1080p micro-OLED AR displays | Waveguide holographic displays |
| Display Resolution | — | 1920 × 1080 per eye | 2048 × 1080 per eye |
| Field of View (FOV) | — | ~40° diagonal | ~52° diagonal |
| Optics | — | 45% transparency | Diffractive waveguides |
| Brightness | — | ~200–400 nits | ~500 nits equivalent |
| Cameras | 12 MP photo, 1080p60 video | 8-MP RGB + dual fisheye tracking | Depth camera + 8-MP RGB |
| Sensors | Accelerometer, gyroscope, touch panel, mics | IMU, tracking cameras | IMU, depth sensor, IR cams, eye tracking |
| Tracking Type | No positional tracking | 6DoF inside-out | 6DoF inside-out |
| Hand Tracking | No | Yes (basic) | Yes (advanced) |
| Eye Tracking | No | No | Yes |
| Environment Mapping | No | Spatial mapping | Real-time spatial mapping |
| Audio | Open-ear speakers; 5-mic array | Stereo speakers | Spatial sound speakers |
| Voice Assistant | Meta AI | No | Cortana (enterprise optional) |
| Connectivity | Bluetooth 5.x | USB-C tether | Wi-Fi 5, Bluetooth |
| Battery Life | 3–4 hours | Tethered (no battery) | 2–3 hours |
| Charging Time | ~1 hour | — | 65–90 minutes |
| Battery Capacity | Not specified | — | ~15.2 Wh |
| Controller Support | None | Optional 6DoF controller | Hand tracking only |
| Compute Mode | On-device | Tethered | Standalone |
| Software Ecosystem | Meta apps | ThinkReality platform | Dynamics 365, MR apps |
| Development Platform | Meta SDK | ThinkReality SDK | Unity, Unreal, MRTK |
| Security / Privacy | Capture LED, encrypted pairing | Enterprise controls | Enterprise security, secure boot |
| Use Cases | Social content, music, calls | Enterprise AR workflows | MR training, surgery, engineering |
| Build Material | Acetate frame | Plastic/metal | Carbon fiber, plastic |
| Comfort / Fit | Everyday sunglasses | Light but tethered | Balanced head-mounted |
| Environmental Rating | — | Depends on PC/phone | IP50 |
| Accessories | Charging case | Safety frame, PC/phone | Fit kit, carry case |
| Supported Platforms | iOS, Android | PC, Motorola phones | Windows |
| Developer Access | Moderate | Moderate | Full enterprise SDK |
| Safety Features | Recording LED | Enterprise admin controls | Eye-safety compliance |